Assignment Questions

1. Mutex Assignment

Assignment: Protect a Shared Device Buffer Using Mutex

Problem Statement

Write a Linux character device driver that maintains an internal kernel buffer. The driver should support open(), read(), write(), and release() operations.

Multiple user processes may access the device at the same time. Use a mutex to protect the shared buffer from race conditions during read() and write().

Requirements

Implement:

open()

read()

write()

release()

Use:

struct mutex buffer_lock;

mutex_init(&buffer_lock);

mutex_lock(&buffer_lock);

mutex_unlock(&buffer_lock);

Expected Behavior

- When user writes data to the device, the data should be stored in the kernel buffer.

- When user reads from the device, the stored data should be copied to user space.

- Simultaneous read/write access should be protected using a mutex.

Testing Commands

echo "Hello Kernel" | sudo tee /dev/mutex_dev

cat /dev/mutex_dev

Concepts Tested

- Process context synchronization

- Race condition prevention

- Safe access to shared buffer

- mutex_lock() and mutex_unlock()

Important Question

Why is mutex suitable here instead of spinlock?

2. Semaphore Assignment

Assignment: Limit Device Access to Three Users Using Counting Semaphore

Problem Statement

Write a Linux character device driver where only three processes can open the device at the same time.

Use a counting semaphore initialized with value 3.

Requirements

Use:

struct semaphore sem;

sema_init(&sem, 3);

down_interruptible(&sem);

up(&sem);

In open():

if (down_interruptible(&sem))

return -ERESTARTSYS;

In release():

up(&sem);

Expected Behavior

- First three processes should be able to open the device.

- Fourth process should block until one of the first three processes closes the device.

- When a process closes the device, semaphore count should increase.

Testing Commands

Open multiple terminals and run:

sudo cat /dev/semaphore_dev

Concepts Tested

- Counting semaphore

- Resource limiting

- Blocking synchronization

- Difference between mutex and semaphore

Important Question

Why is semaphore better than mutex when more than one process should be allowed to access a resource?

3. Low-Resolution Kernel Timer Assignment

Assignment: Periodic Counter Using Low-Resolution Kernel Timer

Problem Statement

Write a kernel module using a low-resolution kernel timer. The timer should expire every 1 second and increment a global counter.

The counter value should be readable from a character device.

Requirements

Use Linux kernel timer APIs:

struct timer_list my_timer;

timer_setup(&my_timer, timer_callback, 0);

mod_timer(&my_timer, jiffies + msecs_to_jiffies(1000));

del_timer_sync(&my_timer);

Expected Behavior

- Timer starts when module is inserted.

- Timer callback runs every 1 second.

- Each callback increments timer_count.

- Reading the device should show the current value of timer_count.

Testing Commands

sudo insmod low_timer.ko

cat /dev/low_timer_dev

sleep 3

cat /dev/low_timer_dev

sudo rmmod low_timer

Expected Output Example

timer_count = 1

timer_count = 4

Concepts Tested

- jiffies

- timer_list

- Timer callback function

- Periodic timer rearming using mod_timer()

- Low-resolution timing behavior

Important Question

Why are low-resolution timers not suitable for highly accurate microsecond-level delays?

4. High-Resolution Kernel Timer Assignment

Assignment: Accurate Periodic Event Using High-Resolution Timer

Problem Statement

Write a kernel module using a high-resolution timer. The timer should expire every 500 milliseconds and increment a global counter.

The counter value should be readable from a character device.

Requirements

Use high-resolution timer APIs:

struct hrtimer my_hrtimer;

ktime_t interval;

hrtimer_init(&my_hrtimer, CLOCK_MONOTONIC, HRTIMER_MODE_REL);

my_hrtimer.function = hrtimer_callback;

hrtimer_start(&my_hrtimer, interval, HRTIMER_MODE_REL);

hrtimer_forward_now(&my_hrtimer, interval);

hrtimer_cancel(&my_hrtimer);

Timer callback should return:

return HRTIMER_RESTART;

Expected Behavior

- High-resolution timer starts when module is inserted.

- Timer expires every 500 ms.

- Each expiry increments hrtimer_count.

- Reading the device should show the current count.

Testing Commands

sudo insmod high_timer.ko

cat /dev/high_timer_dev

sleep 2

cat /dev/high_timer_dev

sudo rmmod high_timer

Expected Output Example

hrtimer_count = 1

hrtimer_count = 5

Concepts Tested

- hrtimer

- ktime_t

- CLOCK_MONOTONIC

- High-resolution periodic timing

- Difference between timer_list and hrtimer

Important Question

Why is hrtimer preferred when better timing precision is required?

Summary Table

| Topic | Assignment Focus | Main API |
| --- | --- | --- |
| Mutex | Protect shared buffer | mutex_lock(), mutex_unlock() |
| Semaphore | Limit access to 3 users | down_interruptible(), up() |
| Low-resolution timer | Periodic 1-second counter | timer_list, mod_timer() |
| High-resolution timer | Periodic 500 ms counter | hrtimer_start(), hrtimer_forward_now() |